بسم الله الرحمن الرحيم

## King Saud University

College of Engineering

Civil Engineering Department

#### SE 423: “Digital Images Processing”

1444 3rd Semester

### Homework #3 Solutions

Q1: What would be the advantages of geometrically correcting an image to 		      geographic coordinates prior to further analysis & interpretation?

Answer;

The goal of image rectification & restoration is to correct image data for distortions or degradations which stems from the image acquisition process. The goal of geometric corrections is to compensate for the distortions introduced by these errors so that the corrected image will have the geometric integrity of a map screen. Therefore, the analyzed data will be tied to the right geographic location.

Q2: Assume that you are trying to register an OLI Landsat-8 satellite distorted 	scene to a SPOT-7 satellite panchromatic scene of the same area:

- What is the minimum number of control points to perform:

Answer; Base on the minimum # of GCP Law = (t+1)*(t+2) / 2.			Where “t” is the order of the polynomial;

- An affine transformation. 			(3 GCP’s)

- A second degree polynomial transformation. 	(6 GCP’s)

- A third degree polynomial transformation. 	(10 GCP’s)

- Why more points are usually measured?

Answer; To be apple to make a least square adjustment to increase the accuracy & to guaranty a good distribution of the GCPs.

- What are the advantages & disadvantages of using higher 	degree polynomial?

Answer; It could increase the accuracy but it will increase the number of the GCPs.

Q 3: In Figure 1, resample the output pixel (r) using the nearest neighbor & the 	       bilinear resampling methods.

60			  80

0.2

0.4   r

40			  50

Figure 1.

Answer;

The nearest neighbor to the output pixel r is the value 60 in the 	upper left corner, therefore the DNr = 60, according to this method.

For the bilinear resampling method, the value of DNr can be 	computed as follow:

DNm = [DN3 – DN1] * dy + DN1

= (40 – 60) * 0.2 + 60 = 56,

DNn = [DN4 – DN2] * dy + DN2

= (50 – 80) * 0.2 + 80 = 74,

DNr = [DNn – DNm] * dx + DNm

= (74 – 56) *0.4 + 56 = 63.2 ≈ 63