بسم الله الرحمن الرحيم

## King Saud University

College of Engineering

Civil Engineering Department

#### SE 423: “Digital Images Processing”

1444H 3rd Semester

### Homework #4 Solutions

Q1: Explain briefly the different forms of image distortion.

Answer;

The generic types of corrections can be categorized in one of the three types below:

- Geometric distortion. Raw digital image usually contain geometric distortions so significant that it cannot be used as a map. These errors can be either of systematic or random nature. The goal of geometric corrections is to compensate for the distortions introduced by these errors so that the corrected image will have the geometric integrity of a map screen.

- Radiometric distortion. The radiance measured by any given system over a given object is influenced by such features as change in scene illumination, atmospheric conditions, viewing geometry & instruments response characteristics. The need to perform correction for any or all of the radiometric influences depends directly upon the particular application at hand.

- Noise distortion. Image noise is any unwanted disturbance in image data that is due to limitations in sensing, signal digitization, or data recording process. The potential sources of noise range from periodic drafts or malfunction of a detector to electronic interference between sensor components. The correction required depends upon whether the noise is systematic (periodic), random, or some combination of the two.

Q2: Explain briefly the different forms of image noise corrections.

Answer;

The potential sources of noise range from periodic drafts or malfunction of a detector to electronic interference between sensor components. The correction required depends upon whether the noise is periodic, random, or some combination of the two.

First, Periodic or Line-Oriented Noise Removal. This kind of noise, radiometric response of one, or more, detectors tend to drift over time resulting in relatively higher or lower values along every certain lines. Two forms of line-oriented noise problems, namely; line drop, line striping:

- Line Drop. In this types of problems, a number of adjacent pixels along a line can contain spurious DNs. To restore these lines, we have to calculate a replacement for the spurious DNs. To do this, the average DN values for each line for the entire scene is computed & compared to the overall average of the scene. A line is identification defected if it is deviating from the overall average by more than a designed threshold value. Next, defected lines are fixed by taken the average value of the pixels above & below each defected pixel on the defected line.

- Line Striping. Valid data are present in this kind of line oriented problem; however, the defected lines have to be corrected to match the overall scene. To fix this problem, we first to compute a correction value by subtracting the average DNs for the entire scene from the average DNs for the defected line. Then subtract this value from each value in the defected line.

Second, Random Noise Removal. This type of noise is characterized by non-systematic variations in DNs from pixel to pixel called bit errors. Such errors are often referred to as being “spikey” in character & it can cause a “salt & paper” appearance. Random noise can be handled by recognizing that noised values normally change much more abruptly than the rest of the image values. They can be fixed by applying 3x3 digital filters, such as: (1) a 3x3 average digital filter, or (2) a 3x3 average digital filter with double the weight in the center, (3) or simply we can take the median of a 3x3 digital filter.

Q3: Remove all possible form of noise in the scene given in Figure 1.

PIXELS

|  |  |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L | 0 | 90 | 90 | 90 | 90 | 90 | 90 | 80 | 90 | 80 | 80 |
|  |  | I | 1 | 20 | 30 | 40 | 50 | 40 | 50 | 50 | 50 | 50 | 40 |
|  |  | N | 2 | 20 | 90 | 30 | 70 | 60 | 60 | 80 | 50 | 30 | 50 |
|  |  | E | 3 | 20 | 20 | 50 | 60 | 10 | 80 | 70 | 30 | 40 | 50 |
|  |  | S | 4 | 30 | 60 | 60 | 80 | 80 | 70 | 40 | 30 | 50 | 80 |
|  |  |  | 5 | 50 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 50 | 60 |
|  |  |  | 6 | 40 | 60 | 70 | 60 | 70 | 70 | 50 | 30 | 60 | 70 |

Figure 1.

Answer;

There are 3 form of distortions that need to be fixed:

- Line stripping in line # 0,

- Random noise in pixel (2, 1) & pixel (3, 4), &

- Line drop in line # 5.

First, line stripping in line # 0:

Average DNs of the distorted line = (90+90+ … +80+80) / 10 = 87,

Scene average = (90+90+ … +60+70) / 70 = 51.1 ≈ 51

Correction value = 87 - 51 = 36,

Corrected values; 90 - 36 = 54, & 80 - 36 = 44.

Second, the random noise in pixel (2, 1) & pixel (3, 4):

- With average weight of 3x3 window;

Pixel (2, 1), DN11 = [20+30+40+20+ 90 +30+20+20+50] / 9 			≈ 36,

Pixel (3, 4), DN13 = [70+60+60+60+ 10 +80+80+80+70] / 9 				≈ 63,

- With double the weight in the center:

Pixel (2, 1), DN11 = [20+30+40+20+ (2*90) +30+20+20+50] / 10 			= 41,

Pixel (3, 4), DN13 = [70+60+60+60+ (2*10) +80+80+80+70] / 10 				= 58,

Third line drop in line # 5:

Corrected values; (60+60) / 2 = 60, (60+70) / 2 = 65, (80+60) / 2 = 70, (80+70) / 2 = 75, (70+70) / 2 = 70, (40+50) / 2 = 45, & (30+30) / 2 = 30.

PIXELS

|  |  |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L | 0 | 54 | 54 | 54 | 54 | 54 | 54 | 44 | 54 | 44 | 44 |
|  |  | I | 1 | 20 | 30 | 40 | 50 | 40 | 50 | 50 | 50 | 50 | 40 |
|  |  | N | 2 | 20 | 36 or 41 | 30 | 70 | 60 | 60 | 80 | 50 | 30 | 50 |
|  |  | E | 3 | 20 | 20 | 50 | 60 | 63 or 58 | 80 | 70 | 30 | 40 | 50 |
|  |  | S | 4 | 30 | 60 | 60 | 80 | 80 | 70 | 40 | 30 | 50 | 80 |
|  |  |  | 5 | 50 | 60 | 65 | 70 | 75 | 70 | 45 | 30 | 50 | 60 |
|  |  |  | 6 | 40 | 60 | 70 | 60 | 70 | 70 | 50 | 30 | 60 | 70 |

Figure 1: Corrected 1-band 7x10 image.