بسم الله الرحمن الرحيم

## King Saud University

College of Engineering

Civil Engineering Department

#### SE 423: “Digital Images Processing”

1444H 3rd Semester

### Homework #5 solutions

Q1: In Figure-1, Split the OLI2 band into water & non-water bodies after 	you find a suitable gray level threshold in the OLI5 band:

|  | 0 | 1 | 2 | 3 |  |  |  | 0 | 1 | 2 | 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 30 | 50 | 30 | 50 |  |  |  | 20 | 20 | 30 | 70 |  |
| 1 | 40 | 40 | 50 | 60 |  |  |  | 50 | 15 | 20 | 55 |  |
| 2 | 40 | 60 | 60 | 50 |  |  |  | 60 | 55 | 15 | 70 |  |
| 3 | 40 | 40 | 50 | 30 |  |  |  | 10 | 20 | 65 | 10 |  |

OLI2						        OLI5

Figure-1.

Answer;

| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  | * |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  | * |  |  |  |  |  |  |  |  |  |  |
| 2 | * | * | * |  |  |  |  |  |  | * |  |  | * |
| 1 | * | * | * |  | * |  |  |  | * | * | * | * | * |
|  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 55 | 60 | 65 | 70 |
|  |  |  |  | OLI5 histogram |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 |  | 0 | 1 | 2 | 3 |  |  |  |  |
| 0 | 30 | 50 | 30 | 0 |  | 0 | 0 | 0 | 50 |  |  |  |  |
| 1 | 0 | 40 | 50 | 0 |  | 40 | 0 | 0 | 60 |  |  |  |  |
| 2 | 0 | 0 | 60 | 0 |  | 40 | 60 | 0 | 50 |  |  |  |  |
| 3 | 40 | 40 | 0 | 30 |  | 0 | 0 | 50 | 0 |  |  |  |  |

Water bodies			Non-water bodies

# Q2: For the image in Figure 2, stretch the image linearly between 0, & 	100:

# Before noise removal,

- After noise removal using double the weight in the center.

|  |  | 50 | 50 | 40 | 40 | 40 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 | 10 | 50 | 90 | 40 |  |  |
|  |  | 60 | 60 | 50 | 50 | 50 |  |  |

Figure 2.

Answer;

- Before noise removal;

MIN = 10, MAX = 90, Range = 100;

DN’ = (DN – 10) / (90 –10) * 100

|  | DN | DN’ |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 0 |  | 50 | 50 | 38 | 38 | 38 |
|  | 40 | 38 |  | 50 | 0 | 50 | 100 | 38 |
|  | 50 | 50 |  | 63 | 63 | 50 | 50 | 50 |
|  | 60 | 63 |  |  |  |  |  |  |
|  | 90 | 100 |  |  |  |  |  |  |

- After noise removal using double the weight in the center:

DN10 = (50+50+40+50+2*10+50+60+60+50)/10 = 43

DN100 = (40+40+40+50+2*90+40+50+50+50)/10 = 54

MIN = 40, MAX = 60, Range = 100;

DN’ = (DN – 40) / (60 –40) * 100

|  | DN | DN’ |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 40 | 0 |  | 50 | 50 | 0 | 0 | 0 |
|  | 43 | 15 |  | 50 | 15 | 50 | 70 | 0 |
|  | 50 | 50 |  | 100 | 100 | 50 | 50 | 50 |
|  | 54 | 70 |  |  |  |  |  |  |
|  | 60 | 100 |  |  |  |  |  |  |

Q3: Stretch the histogram image given in Table 1, linearly, linearly 	between 5 & 12, & nonlinear stretch:

#### Table 1

|  |  | DN | ##### Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 0 | 0 |  |  |
|  |  | 1 | 0 |  |  |
|  |  | 2 | 100 |  |  |
|  |  | 3 | 150 |  |  |
|  |  | 4 | 250 |  |  |
|  |  | 5 | 790 |  |  |
|  |  | 6 | 300 |  |  |
|  |  | 7 | 1023 |  |  |
|  |  | 8 | 850 |  |  |
|  |  | 9 | 656 |  |  |
|  |  | 10 | 329 |  |  |
|  |  | 11 | 245 |  |  |
|  |  | 12 | 122 |  |  |
|  |  | 13 | 104 |  |  |
|  |  | 14 | 81 |  |  |
|  |  | 15 | 0 |  |  |
|  |  | 16 | 0 |  |  |
|  |  | 17 | 0 |  |  |
|  |  | 18 | 0 |  |  |
|  |  | 19 | 0 |  |  |
|  |  | 20 | 0 |  |  |

Answer;

- Linear stretch;					ii)  Linear stretch;

MIN = 2, MAX = 14, Range = 20; 	      MIN = 5, MAX = 12, Range = 20;

DN’ = (DN – 2) / (14 –2) * 20		      DN’ = (DN – 5) / (12 –5) * 20

- Table-1 (i), solution				     Table-2, (ii), solution

|  | DN | DN’ |  | DN | DN’ |
| --- | --- | --- | --- | --- | --- |
|  | 0-2 | 0 |  | 0-5 | 0 |
|  | 3 | 2 |  | 6 | 3 |
|  | 4 | 3 |  | 7 | 6 |
|  | 5 | 5 |  | 8 | 9 |
|  | 6 | 7 |  | 9 | 11 |
|  | 7 | 8 |  | 10 | 14 |
|  | 8 | 10 |  | 11 | 17 |
|  | 9 | 12 |  | 12-20 | 20 |
|  | 10 | 13 |  |  |  |
|  | 11 | 15 |  |  |  |
|  | 12 | 16 |  |  |  |
|  | 13 | 18 |  |  |  |
|  | 14-20 | 20 |  |  |  |

- Nonlinear stretch (10 points);

Total number of pixels = 5000 pixels, Range = 20 values,

Average number of pixels per value = 5000 / 20 = 250 pixel,

- Table-1 (b), solution

|  | DN | DN’ | Frequency | ratio | Increments |
| --- | --- | --- | --- | --- | --- |
|  | 0-3 | 1 | 250 | 1 | 1 |
|  | 4 | 2 | 250 | 1 | 1 |
|  | 5 | 3 | 790 | 3.16 | 3 |
|  | 6 | 6 | 300 | 1.20 | 1 |
|  | 7 | 7 | 1023 | 4.11 | 4 |
|  | 8 | 11 | 850 | 3.40 | 3 |
|  | 9 | 14 | 656 | 2.62 | 3 |
|  | 10 | 17 | 329 | 1.32 | 1 |
|  | 11 | 18 | 245 | 0.98 | 1 |
|  | 12-13 | 19 | 226 | 0.90 | 1 |
|  | 14-20 | 20 | 81 | 0.32 | 0 |