بسم الله الرحمن الرحيم

## King Saud University

College of Engineering

Civil Engineering Department

#### SE 423: “Digital Images Processing”

1444H 3rd Semester

### Homework #6 Solutions

Q1; Find out the output of the edge detector running, horizontal 		first difference, vertical first difference, diagonal first 			difference & cross diagonal first difference on the input 		image of Figure 1:

# Input image						  Kernel

1 1 1 1 1 1 9 9					  A	H

1 1 1 1 1 9 9 9					  V	D

1 1 1 9 1 9 9 9					 Figure 2.

1 1 9 9 1 9 9 1

1 9 9 9 1 9 9 1

9 9 1 9 1 9 1 1

Figure 1.

Answer;

- horizontal first difference; |DNa – DNh|

| 1 | 1 | 1 | 1 | 1 | 1 | 9 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 9 | 9 | 9 |
| 1 | 1 | 1 | 9 | 1 | 9 | 9 | 9 |
| 1 | 1 | 9 | 9 | 1 | 9 | 9 | 1 |
| 1 | 9 | 9 | 9 | 1 | 9 | 9 | 1 |
| 9 | 9 | 1 | 9 | 1 | 9 | 1 | 1 |

For the first window, shown in yellow, applied on the first line, DNa = 1, & 	DNh = 1 => |1 – 1| = 0,

This is also true for the next 4 windows: however, the 6th window, shown in blue, applied also on the first line, DNa = 1, & DNh = 9 => |1 – 9| = 8,

For the 3rd window, shown in green, applied on the 3rd line, DNa = 1, &	DNh = 9 => |1 – 9| = 8,

For the 7th window, shown in gray, applied on the 5th line, DNa = 9, & 	DNh = 1 => |9 – 1| = 8,

The overall result after applying the 2x2 horizontal first difference window is as fallow:

| 0 | 0 | 0 | 0 | 0 | 8 | 0 | 9 | The high difference value, 8 here, represent an edge, while the 0’s represent no edges, the last column stays as is because we cannot apply the window on it because DNh has no value. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 8 | 0 | 0 | 9 |  |
| 0 | 0 | 8 | 8 | 8 | 0 | 0 | 9 |  |
| 0 | 8 | 0 | 8 | 8 | 0 | 8 | 1 |  |
| 8 | 0 | 0 | 8 | 8 | 0 | 8 | 1 |  |
| 0 | 8 | 8 | 8 | 8 | 8 | 0 | 1 |  |

- vertical first difference; |DNa – DNv|

For the first window, shown in yellow, applied on the first line, DNa = 1, & 	DNv = 1 => |1 – 1| = 0,

This is also true for the next 4 windows: however, the 6th window, shown in blue, applied also on the first line, DNa = 1, & DNv = 9 => |1 – 9| = 8,

For the 3rd window, shown in green, applied on the 3rd line, DNa = 1, & 	DNv = 9 => |1 – 9| = 8,

For the 7th window, shown in gray, applied on the 5th line, DNa = 9, & 	DNv = 1 => |9 – 1| = 8,

The overall result after applying the 2x2 horizontal first difference window is as fallow:

| 0 | 0 | 0 | 0 | 0 | 8 | 0 | 0 | The high difference value, 8 her, represent an edge, while the 0’s represent no edges, the last line stays as is because we cannot apply the window on it because DNv has no value. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 8 | 0 | 0 | 0 | 0 |  |
| 0 | 0 | 8 | 0 | 0 | 0 | 0 | 8 |  |
| 0 | 8 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| 8 | 0 | 8 | 0 | 0 | 0 | 8 | 1 |  |
| 9 | 9 | 1 | 9 | 1 | 9 | 1 | 1 |  |

- diagonal first difference; |DNa – DNd|

For the first window, shown in yellow, applied on the first line, DNa = 1, & 	DNd = 1 => |1 – 1| = 0,

This is also true for the next 3 windows: however, the 6th window, shown in blue, applied also on the first line, DNa = 1, & DNd = 9 => |1 – 9| = 8,

For the 3rd window, shown in green, applied on the 3rd line, DNa = 1, & 	DNd = 9 => |1 – 9| = 8,

For the 7th window, shown in gray, applied on the 5th line, DNa = 9, & 	DNd = 1 => |9 – 1| = 8,

The overall result after applying the 2x2 horizontal first difference window is as fallow:

| 0 | 0 | 0 | 0 | 8 | 8 | 0 | 9 | The high difference value, 8 her, represent an edge, while the 0’s represent no edges, the last line & last column stay as they are because we cannot apply the window on them because DNd has no value. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 8 | 0 | 8 | 0 | 0 | 9 |  |
| 0 | 8 | 8 | 8 | 8 | 0 | 8 | 9 |  |
| 8 | 8 | 0 | 8 | 8 | 0 | 8 | 1 |  |
| 8 | 8 | 0 | 8 | 8 | 8 | 8 | 1 |  |
| 9 | 9 | 1 | 9 | 1 | 9 | 1 | 1 |  |

- cross diagonal first difference; |DNa – DNd| + |DNv – DNh|

For the first window, shown in yellow, applied on the first line, DNa = 1, DNh 	= 1, DNv = 1, DNd = 1 => |1 – 1| + |1 – 1| = 0,

This is also true for the next 3 windows: however, the 6th window, shown in blue, applied also on the first line, DNa = 1, DNh = 9, DNv 	= 9, DNd = 9 => 	 |1 – 9| + |9 – 9| = 8,

For the 3rd window, shown in green, applied on the 3rd line, DNa = 1, DNh = 9, 	DNv = 9, DNd = 9 => 	 |1 – 9| + |9 – 9| = 8,

For the 7th window, shown in gray, applied on the 5th line, DNa = 9, 	DNh = 1, DNv = 1, DNd = 1 => |9 – 1| + |1 – 1| = 8,

The overall result after applying the 2x2 horizontal first difference window is as fallow:

| 0 | 0 | 0 | 0 | 8 | 8 | 0 | 9 | The high difference value, 8 & 16 here, represent an edge, while the 0’s represent no edges, the last line & column stay as they are because we cannot apply the window on them because DNd has no value. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 8 | 8 | 16 | 0 | 0 | 9 |  |
| 0 | 8 | 8 | 16 | 16 | 0 | 8 | 9 |  |
| 8 | 8 | 0 | 16 | 16 | 0 | 16 | 1 |  |
| 8 | 8 | 8 | 16 | 16 | 8 | 8 | 1 |  |
| 9 | 9 | 1 | 9 | 1 | 9 | 1 | 1 |  |

Q2; For the image in band A, band B, & band C Figure 3 do the 			following:

|  | 0 | 1 | 2 | 3 |  | 0 | 1 | 2 | 3 |  | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 35 | 35 | 45 | 45 |  | 30 | 20 | 20 | 30 |  | 50 | 60 | 40 | 70 |
| 1 | 25 | 45 | 45 | 35 |  | 10 | 10 | 20 | 20 |  | 60 | 60 | 40 | 50 |

Band A			      Band B			    Band C

Figure 3.

- Calculate the spectral ratio of A/B for the first line by 2 different methods & compare your results, &

- Find out all possible ratio combination for pixel (1, 3).

- If B represents the red band & C represents N-IR find out the TVI for pixel (1, 1).

Answer;

- Calculate the spectral ratio of A/B for the first line by 2 different methods & compare your results;

First, Angle Ratio;

DN’ = R arctan [DNyi / (DNxi + 1)]

DN’1, 0 = 162.3 arctan [35 / (30 + 1)] ≈137

DN’1, 1 = 162.3 arctan [35 / (20 + 1)] ≈167

DN’1, 2 = 162.3 arctan [45 / (20 + 1)] ≈184

DN’1, 3 = 162.3 arctan [45 / (30 + 1)] ≈157

Second Difference Ratio;

DN’ = [1 + (DNyi - DNxi) / (DNyi + DNxi + 1)] * 127, (for 8-8-bit quantization level)

DN’1, 0 = [1 + (35 - 30) / (35 + 30 + 1)] * 127 ≈137

DN’1, 1 = [1 + (35 - 20) / (35 + 20 + 1)] * 127 ≈161

DN’1, 2 = [1 + (45 - 20) / (45 + 20 + 1)] * 127 ≈175

DN’1, 3 = [1 + (45 - 30) / (45 + 30 + 1)] * 127 ≈ 152

Comparing the result from the two methods shows that they are closed to each other.

- Find out all possible ratio combination for pixel (1, 3);

35/20, 35/50, 20/35, 20/50, 50/35, & 50/20.

- If B represents the red band & C represents N-IR find out the TVI for pixel (1, 1).

TVI = {[(DNN-IR – DNred) / (DNN-IR + DNred)] +0.5}1/2 * 100

TVI = {[(60 – 10) / (60 + 10)] +0.5}1/2 * 100 ≈ 110