بسم الله الرحمن الرحيم

## King Saud University

College of Engineering

Civil Engineering Department

#### SE 365: “Principles of Remote Sensing & Image Interpretation”

1444H 1st Semester

### Homework #1

Solutions

Q1: If a frequency of an electromagnetic wave is 500,000 GHz, what is 	the wavelength of that radiation assuming speed of light = 3 x 108 	m/sec? (Express your answer in m). Find the energy of the 	quantum for this frequency? If this wavelength is a dominant 	wavelength of a blackbody, find the energy radiated by this 	blackbody?

Answer;

- The wave length, = C(3 x 108 m/s) * 106 / (500,000 *109) 					= 0.6 m

- Energy of the quantum for this dominant wavelength for this blackbody

Q = h h = 6.626 x 10-34 (J sec),

Q = 6.626 x 10-34 * (500,000 *109) = 3.313 x 10-19 J

- Dominant wave length for this blackbody m = A / T

m = 2898 (m o k)/ T0k = 0.6 m; T = 4,830ok

- Total radiation per square meters for a blackbody M =  T4,

M = 5.6697 x 10-8 (w m-2 o k-4) (4,8300k)4 = 30,856,637.8 w m-2

Q2: Explain why most RS systems avoid detecting & recording 	wavelength in the UV & blue portions of the spectrum.

Answer;

Rayleigh scattering occurs when particles are very small compared to the wavelength of the radiation. It could causes shorter wavelengths of energy to be scattered much more than longer wavelengths. As sunlight passes through the atmosphere, the shorter wavelengths (i.e. UV & blue of the visible spectrum) are scattered more than the other which cause haze especially in the satellite images.

Q3: What are the best atmospheric conditions for RS in the visible 	portion of the spectrum?

Answer;

Late morning on a sunny, dry day with little clouds and no pollution would be very good for remote sensing in the visible wavelengths.