بسم الله الرحمن الرحيم

## King Saud University

College of Engineering

Civil Engineering Department

#### SE 365: “Principles of Remote Sensing & Image Interpretation”

1444H 1st Semester

### Homework #4 Solutions

Q1: Explained briefly how multispectral scanners work. Also explain 	briefly their advantages & disadvantages compare to photographic 	cameras.

Answers:

Multispectral Scanning Systems, (MSS), employ sensors with narrow field of view 	(i.e. IFOV) that sweeps over the terrain to build up & produce a two-	dimensional image of the surface.

MSS has several advantages over photographic systems;

- They are capable of much higher spectral resolution than photographic systems.

- It is easier to determine the specific amount of energy measured, in MSS because data are recorded electronically.

- The digital recording in MSS systems facilitates transmission of data to receiving stations on the ground & immediate processing of data in a computer environment.

Q2: Explained briefly how thermal scanners work.

Answers:

Remote sensing of energy emitted from the Earth's surface in the thermal IR (3 μm 	to 15 μm). Thermal sensors use photo detectors sensitive to the direct 	contact of photons on their surface, to detect emitted thermal radiation. 	Thermal sensors essentially measure the surface temperature & thermal 	properties of targets.

Q3: Explained briefly the three generic types of R.S. imaging satellites & 	the main differences among them.

Answers:

Three Generic types of R.S. imaging satellites:

- Weather Satellites/Sensors; has high coverage area, low spatial resolution, & Spectral resolution that can interact with the atmosphere

- Marine Observation Satellites/Sensors; has high coverage area, low spatial resolution, & spectral resolution that can interact with water bodies but not	much with the atmosphere.

- Land Observation Satellites/Sensors; has medium to small coverage area, medium to high spatial resolution, & spectral resolution that does not interact much with the atmosphere.

Q4: Explain briefly the Landsat satellite program giving some lights on 	its orbital characteristics.

Answer;

The Landsat program, initiated by NASA in 1967, to be the first Land Observation Satellites program ever. Since then NASA has launched 9 satellites in this program. The Landsat program considered to be from the first group, the relatively large coverage area, small spatial resolution and high multispectral-band group. It was designed as an experiment to test the feasibility of collecting multi-spectral earth observation data from an unmanned satellite platform.

All Landsat satellites are placed in circler, repetitive, near-polar, sun-synchronous orbits. Reduced to 705 km altitude since the Landsat-4, with 98.2 degree inclination & Equatorial crossing time = 10:00 am +/- 15 min. local time.

Q5:  Explain briefly the New Millennium Program (NMP) & make a brief 	comparison between the EO-1 & the Landsat-7.

Answer;

The New Millennium Program (NMP), is designed to identify, develop, & flight validate key instrument & spacecraft technologies that can enable new, more cost effective approaches to conduct science missions in the 21st century.

Landsat-7/EO-1 Comparisons

|  | Landsat-7 | EO-1 |
| --- | --- | --- |
| Instrument(s) | ETM+ (using Across-truck system) | ALI, Hyperon, & LAC (using Along-truck system (push-broom)) |
| Mass | 2200 kg | 425 kg |
| Orbit | 705 km, sun-synchronous, 98.2 degree inclination; Equatorial crossing time = 10:00 am +/- 15 min. | 705 km, sun-synchronous, 98.2 degree inclination; Equatorial crossing time = Landsat-7 + 1 min. |
| Bands; Spectral range (m): Ground Res | .450 - .515: (blue), 30 m .525 - .605: (green), 30 m .630 - .690: (red), 30 m .750 - .900: (N-IR) 30 m 1.55 – 1.75: (M-IR1) 30m 10.40 – 12.50: (T_IR) 60 m 2.09 -2.35: (M-IR2) 30m .520 - .900: (pan), 15 m | .433 - .453: : (blue1), 30 m .450 - .510: : (blue2), 30 m .525 - .605: (green), 30 m .630 - .690: (red), 30 m .775 - .805: (N-IR1) 30 m .845 - .890: (N-IR2) 30 m 1.20 – 1.30: (N-IR3) 30 m 1.55 – 1.75: (M-IR1) 30m 2.08 -2.35: (M-IR2) 30m .480 - .608: (pan), 10 m .850 – 1.60: 250 (atmos. Correction) .400 – 2.50: 30 (WIS = 309 bands; GIS = 233 bands) |

Q6: Explain briefly the major changes on Landsat-8 over Landsat-7.

Answer;

Landsat-8 carries two push-broom, first tried in EO-1, instruments: the Operational Land Imager (OLI), and the Thermal Infrared Sensor (TIRS).

The spectral bands of the OLI sensor, while similar to Landsat 7’s ETM+ sensor, provides enhancement from prior Landsat instruments, with the addition of two new spectral bands, similar to the ones in ALI in EO-1, instruments: a deep blue visible channel (band 1) specifically designed for water resources and coastal zone investigation, and a new infrared channel (band 9) for the detection of cirrus clouds. A new Quality Assurance band is also included with each data product. This provides information on the presence of features such as clouds, water, and snow.

The TIRS instrument collects two spectral bands, with 100m resolution, for the wavelength covered by a single band on the previous TM and ETM+ sensors.