بسم الله الرحمن الرحيم

## King Saud University

College of Engineering

Civil Engineering Department

#### SE 365: “Principles of Remote Sensing & Image Interpretation”

1444H 1st Semester

### Homework #6 Solutions

Q1: Explain briefly the advantages of photography over on-the-ground 	observation?

Answer;

In addition to saving time, effort & money, photography has five advantages over on-the-ground observation:

- Improved vantage point; images gives a view of large areas, enabling us to see earth surface features in their spatial context. (i.e. images permit us to look at the “big picture” in which objects of interest reside).

- Capability to stop action; images can give us a “stop action” view of dynamic conditions, such as flood, moving wildlife populations, traffic, oil spills & fires.

- Permanent recording; images are virtually permanent records of existing conditions on the ground at the time of exposure.

- Broadened spectral sensitivity; films can record over a wavelength range about twice as broad as that of the human eye (0.3 to 0.9 m versus 0.4 to 0.7 m). Multi-spectral scanners can have even wider ranges & can see certain phenomena the human eyes cannot see.

- Increasing spatial resolution & geometric fidelity; especially with aerial photography, with the proper selection of camera, film & flight parameters, we can record more spatial details on a photograph than we can see with the unaided human eyes.

Q2: What are the most important factors that can affect the image 	interpretation process? Also explain briefly other factors that 	should be considered in image interpretation?

Answer;

Success in image interpretation depends upon; (a) the abilities & experiences of the interpreter, (b) the quality of the image, & (c) the equipment used to analyze the image.

Other factors that can effects the level of information extracted from an image are; (1) wavelengths &/or film filter combination, electromagnetic energy selection, through the use of film & filters, affects the amount of information that can be interpreted from an image, (2) temporal aspects of the natural phenomena (more positive results can be achieved by obtaining images at several times during the life cycle), (3) weather (the weather can cause significant short-term changes on an image), & (4) scale of a photograph or resolution of an image (scale of photographs can be divided into three main groups; small scales, 1:50,000 or less, used for resource exploration, medium scale, 1:12,000 to 1:50,000, used for identification & mapping, & large scale, 1:12,000 or larger, used for damage assessments).

Q3: Explain briefly the basic characteristics, of an object, that can be used 	to identify that object in image interpretation.

Answer;

Observing the differences between targets & their backgrounds involves comparing different targets based on any, or all, of the visual elements of shape, size, pattern, tone, texture, shadow, site, association & resolution.

Shape refers to the general form, structure, or outline of individual objects.

Size of objects in an image is a function of scale. It is important to assess the size of a target relative to other objects in a scene, as well as the absolute size, to aid in the interpretation of that target.

Pattern refers to the spatial arrangement of visibly discernible objects.

Tone refers to the relative brightness or color of objects in an image.

Texture refers to the arrangement & frequency of tonal variation in particular areas of an image.

Shadow is also helpful in interpretation as it may provide an idea of the profile & relative height of a target which might make identification of a target easier. However, shadows can also reduce or eliminate interpretation in their area of influence, since targets within shadows are much less discernible from their surroundings.

Site refers to the topographic or geographic location & its particularly important aid in the identification of vegetation types.

Association takes into account the relationship between other recognizable objects or features in proximity to the target of interest.

Resolution of an image always place particular limits on image interpretation because some objects are too small or have too little contrast with their surroundings.

Q4: Explain briefly what do interpretation keys mean? And the different 	types of them.

Answer;

Image interpretation keys are shapes, figures, or diagrams that help the interpreter to evaluate information presented on an image in an organized constructive manners.

There are two types of interpretation keys differentiated by the method of presentation of diagnostic features; (1) selective keys, & (2) elimination keys. Selective keys contain numerous example images with their supporting text. On the other hand, elimination keys are arranged in a formation so that the interpreter proceeds step by step from general to specific.

Q5: Explain briefly the land use & land cover terms, & the main 	differences between them.

Answer;

The term “land cover” relates to the type of feature present on the surface of the earth. Conversely, the term “land use” relates to the human activity or economic function associated with a specific piece of land. But while land cover information can be directly interpreted from appropriate images, information about land use cannot always be inferred directly from land cover.

The size of the minimum area that can be mapped on any particular land cover/land use class depends, to some extent, on the scale & resolution of the image. It also depends on the size of data compilation & the final scale of the land cover/land use map.