بسم الله الرحمن الرحيم

## King Saud University

College of Engineering

Civil Engineering Department

#### SE 365: “Principles of Remote Sensing & Image Interpretation”

1444H 1st Semester

### Homework #7 Solutions

Q1: Explain briefly the elements that should be addressed in image 	interpretation for terrain evaluation. (Illustrate your answer using 	sketches).

Answer;

5-key elements that are studies stereoscopically. These elements are topography, drainage pattern & texture, erosion, image tone, vegetation & land use.

Topography; each landform & bedrock type has its own characteristic topographic form that has typical size & shape.

Drainage pattern &texture; there are 6 common drainage patterns, these are; (1) dendritic drainage pattern, a well-integrated pattern formed by main stream branching freely in all directions in homogenous material such as horizontal bedded sedimentary rock & granite. (2) Rectangular drainage pattern, basically a modified dendritic pattern occurs in flat-laying massive sandstone. (3) Trellis drainage pattern, occurs in areas of folded sedimentary rocks. (4) Radial drainage pattern, typical of volcanoes & domes. (5) Centripetal drainage pattern, occurs in areas of limestone & other depressions. (6) Deranged drainage pattern, aimlessly directed short streams occurs in wetland areas. Similarly; there are two drainage textures, these are; (1) Course-textured drainage pattern, developed at hard massive rocks, such as sandstone, where the soils & rocks have good internal drainage with little surface runoff. (2) Fine-textured patterns, occurs at soft easily eroded rocks, such as shale, where soils & rocks have poor internal drainage & high surface runoff.

Erosion; result from the erosion of unconsolidated materials by runoff, gullies are the smallest drainage features that can be seen on aerial photographs. Gullies can be as small as 1 meter wide & 100 meter long. The gullies shape is a characteristic of the material in which they are formed. Three common gullies shapes; (1) Short V-shape cross-sectioned gullies tend to develop in sand & gravel; (2) U-shape cross-sectioned gullies tend to develop in a silty soils; (3) long-gullies with gently rounded cross-sectioned tend to develop in silty & clay soils.

Image Tone; refers to the relative brightness or color of objects in an image.

Vegetation & land use; differences in vegetation often indicate differences in terrain conditions. However, in many cases vegetation & land use obscure differences in terrain conditions.

Q2: Explain briefly the main differences in image identification between 	horizontal bedded sandstone & granitic rocks.

Answer;

| The principal differences in image identification of granitic rocks versus sandstone can be summarized as follows; |  |  |
| --- | --- | --- |
|  | Granite | Sandstone |
| 1 - Evidences of bedding 2 – Topography 3 - Joint pattern 4-Drainage pattern | None. Variable summit. Rounded cliffs. Rounded micro-features. Irregular joint pattern. Dendritic | Bedded. Forms plateaus. Vertical cliffs. Blocky micro-features. Joint system consisting of 2 or 3 principal directions. Rectangular |

Q3: Explain briefly the main differences in image identification between	sandstone capped plateaus & flood basalt plateaus.

Answer;

| The principal differences in image identification of flood basalt plateaus versus sandstone capped plateaus can be summarized as follows; |  |  |
| --- | --- | --- |
|  | Flood Basalt Plateaus | Sandstone Capped Plateaus |
| 1 - Tone 2 – Rock jointing 3 - Drainage | Dark-toned. Columnar jointing. Cliff edges have a serrated appearance. Principally internal drainage. | Light-toned. Joint system consisting of 2 or 3 principal directions. Canyon & cliff edges. Has a law density modified dendratic pattern. |

Q4: Explain briefly the different types of sand dunes formations.

Answer;

Sand dunes vary in shape, depending on the quantity & particle size of the sand, the strength & direction of the wind, & the vegetation conditions, &

Basic sand dunes shapes;

  - Transverse sand dunes;

    - Tend to form in areas with a large supply of sand & little vegetation, &

    - Often cover large areas & develop a wave-like perpendicular to the direction of the wind.

  - Barchans dunes;

    - Crescent-shaped dunes,

    - Tend to form in areas where there is a limited amount of sand & little vegetation, &

    - The tips of the dunes point downwind.

  - Parabolic sand dunes;

    - Have a crescent shape with long horns pointed into the wind, &

    - Formed where vegetation anchors portions of the horns, allowing the center to migrate.

  - Longitudinal sand dunes;

    - long, nearly straight ridges of sand that form parallel to the direction of the wind, &

    - Symmetrical with a width several times the height.