بسم الله الرحمن الرحيم

## King Saud University

College of Engineering

Civil Engineering Department

#### SE 365: “Principles of Remote Sensing & Image Interpretation”

1444H 1st Semester

### Homework #8 Solutions

Q1: Explain briefly how image interpretation can be used to assess & 	monitor water & water quality.

Answer;

Image interpretation can be used in a variety of ways to assess & monitor surface & near surface water quality & distribution.  Most of the sunlight that enter a clear water body is absorbed within about 2-meter of the surface.  The degree of absorption is highly dependent on the wavelength.  Near-IR wavelengths, for example, are absorbed in only a few centimeters resulting in a very dark image tones, (we can use this fact to identify surface water bodies very easily).  The best sunlight penetration is achieved between the wavelengths of 0.48 & 0.60 m (i.e., long-blue & green lights).  Polluted water with impurities, however, tends to reflect more lights in the UV, near-IR, thermal-IR, & radar portions of the spectrum depending on the impurity type.  For example, sedimentation runoff & sewage outfalls will reflect high values in Near-IR wavelengths resulting in a sharp contrast in the image ton between clear & polluted water.  Hot water, on the other hand, will reflect high values in the thermal-IR.  Oil slicks on sea water can be detected using UV, thermal-IR, & radar imager, etc, (using these images we can recognize pollutant & polluted water in a very fast & accurate ways).

For near surface water we could use image interpretation to identify land formations that are most appealing to host underground water. Also we could use these methods to choose the most adequate places for constructing dams & reservoirs.  Finally, we could use image interpretation to monitor & protect recharging zones from pollutant that could harm the underground water supplies.

Q2: Explain briefly how image interpretation can be used to assess	damages caused by water floods.

Answer;

Remote Sensing & Image Interpretation technology along with Geospatial Information System (GIS) has become the key tool for flood monitoring in recent years. The central focus in this field revolves around delineation of flood zones & preparation of flood hazard maps for the vulnerable areas. Digital elevation model (DEM) is considered to be the most effective means to estimate flood depth from remotely sensed or hydrological data.

Because disasters & emergencies often have fast changing, broad impact on a large spatial extent, decision-makers require the rapid re-characterizing of the area using Remote Sensing & Image Interpretation techniques. They help document the need for disaster relief & can be utilized by insurance agencies to assist in assessing the value of property losses.

Flood damage assessment across large areas can be facilitated by the use of satellite images; however, exact information has to be estimated by large-scale photographs.

Q3: Explain briefly how image interpretation can be used to protect the 	environment & ground water supplies.

Answer;

The environment virtually involves everything in the world around us; this includes, natural, physical, biotic, abiotic & human socio-economic features.

Real-time imaging is effectively used to monitor the spillage of hazardous materials, vegetation damage, & threats to natural drainage & human welfare.

Image interpretation may also be used to help locate potential sites for drilling & sampling of hazardous wastes.

Stereoscopic viewing is also impotent because it helps the analysis to identify slopes, relief, & direction of surface drainage.

Remote Sensing & GIS are the only technique that can provide complete approach to the study of total environment while still make visible the different interrelationships that exist within the different biophysical components.

It is also important to be able to identify ground water recharging zones in order to protect them from activities that would pollute the groundwater supply.

Q4: Explain briefly how image interpretation can be used to monitor & 	control the horizontal spread of urban areas.

Answer;

Urban & regional planning can be defined as the orderly regulation of the physical facilities of a city to meet the changing economic & social need of a community including the development of plans for future industrial expansion.

The change in land use from rural to urban is monitored to estimate populations, predict & plan direction of sprawl for developers.

Multi-temporal images provide city administrators with a complete, continuous, cost-effective, view of the city & its environs.

Image Interpretation can assist in several of urban & regional planning activities such as, population estimates, housing quality, & traffic studies, etc.

Q5: Explain briefly the main steps involved in oil exploration & how 	image interpretation can be employed in the process.

Answer;

The search for oil in unexplored onshore areas normally follows a pattern, beginning from regional reconnaissance & culminates with drilling wildcat wells. A typical exploration program proceeds as follows;

- Regional R.S. reconnaissance surveys to locate sedimentary basins that are essential for the formation of oil fields.

Low resolution images with multi-spectral sensitivities are used to identify possible oil fields. SAR images collected at different times can help eliminate terrain features.

- Reconnaissance geophysical surveys. Aerial magnetic surveys are made to produce maps that record the intensity of the earth’s magnetic fields. (Sedimentary basins have lower magnetic intensities).

- Detailed image interpretation, both aerial & space images are used, 3-D stereoscopic images are recommended.

Satellite images are used to identify manmade & terrain features & update their coordinates. 3D terrain visuals are used to identify map rock formations, elevation & major structures. Also they are used to differentiate major rock types & identify barren & productive basin areas.

- Seismic surveys, explosives or mechanical devices are used to transmit waves to show what is present underneath the earth's surface.

The seismic wave travels into the earth & it is reflected by subsurface formations. Then, it is recorded by receivers.

- Wildcat wells drilling to estimate the oil production feasibilities.

Wildcat drilling is a form of exploratory drilling that seeks to exploit unproven or high-risk areas.