بسم الله الرحمن الرحيم

## King Saud University

College of Engineering

Civil Engineering Department

#### SE 365: “Principles of Remote Sensing & Image Interpretation”

1444H 1st Semester

### Homework #9 Solutions

Q1: Explain briefly why we process image data digitally.

Answer;

To keep up with the advancement in the satellite imaging, we have to process image data digitally because we need to process images with speed & accuracy using the computer. The problem is that the computer can only deal with data on digital format.

So why do we need the computer in image processing?

Despite the fact that humans are faster in recognizing images, computers are needed because:

- Computers are much more adept at storing & manipulating huge amount of data with speed & accuracy.

- There are certain thresholds beyond which the human interpreter cannot detect.

- Results obtained by a computer are always repeatable, even when they are wrong!

Q2: Explain briefly the different ways of converting hard copy 	photographs	into digital images?

Answer;

The gray shades, or brightness values, in such image may be converted into discrete values that are proportional to the integrated output of a small pixel. This process is called analog-to-digital (A-to-D) conversion & creates a matrix of brightness values corresponding to the average radiance measured from individual pixels that have been scanned.

Three methods for converting hard-copy photography into a format suitable for digital image processing:

- Optical-Mechanical Scanning:

Flat-Bed & Rotating-Drum Scanners are two types of optical-mechanical scanning systems that are used to convert hard-copy photographic images into digital values. Both of these systems measure image density with an instrument called a densitometer.

A densitometer is a device that measures the average density of a small area of specified size on a photographic transparency or print.

This process operates on a pixel-by-pixel basis & results in a matrix of values that are usually recorded in 8 bit quantization level ranging from 0 to 255.

- Flat-Bed & Rotating-Drum microdensitometers are relatively slow, expensive, & difficult to maintain.

- But offer repeatable spatial & radiometric accuracy.

- Video Digitization:

Another approach to digitizing hard-copy imagery can be through the use of a video camera. Video digitizing involves “freezing” & then digitization of individual video frames.

- Video digitizing represents a low-cost alternative which is convenient.

- But exhibits generally poor spatial resolution & often unrepeatable radiometric or gray-scale resolution.

- Linear photodiode, or charged-coupled-device (CCD) digitization:

CCDs are very small detectors designed to rapidly encode images in a numeric format for subsequent computer analysis.

- Typical output from such systems is a matrix with 1 to 12 bits per pixel & results in high spatial resolution, good radiometric accuracy, & fast digitization.

- The only drawback is that it has a relatively high initial coast.

Q3: Explain briefly what is needed to design a digital image processing 	system.

Answer;

When designing a digital image processing system, the following factors should be considered in the following sequence:

- The maximum number of access to the system at one time.

- Softwares necessary for the digital image processing system.

- Hardware considerations, such as the CPU, the RAM, operating systems & compilers, the amount & types of mass storage required, the spatial & color resolutions of the monitor.

In addition to the above requirements, one should consider other necessary supporting equipments such as printers, plotters, digitizers, scanners, etc.

Q4: Explain briefly the five broad types of computer assisted operations 	in digital image processing.

Answer;

Digital image processing is an extremely broad subject that often involves procedures that can be mathematically complex. All of the procedures involved in digital image processing operations may be categorized into one or more of the following five broad types of computer assisted operations:

- Image Rectification & Restoration, or “pre-processing”.

The aim of these operations is to correct distorted or degraded image data to create a more faithful representation of the original scene.

- Image Enhancement.

These procedures are applied to image data in order to more effectively display the data for subsequent visual interpretation.

- Image Classification.

The objective of these operations is to replace visual analysis of the image data with quantitative techniques for automating the identification of features in a scene.

- Data Merging.

Data merging are used to combine image data for a given geographic area with other geographically referenced data sets for the same area.

- Biophysical Modeling.

The objective of these procedures is to relate quantitatively the digital data recorded by a R.S. system to 	biophysical features & phenomena measured on the ground, such as water depth & pollution considerations.

Nonetheless, we should always remember that the bounders between these various operations are not well defined in practice.